### Урок 2. Обзор доступных сенсоров. АЦП. Считывание данных. Получение данных с потенциометра и оценка угла поворота его ручки. Простая фильтрация

Задание 1. Необходимо подключить потенциометр и несколько светодиодов. Затем написать программу, в которой крутя ручку потенциометра, мы включали и выключали светодиоды. Например: светодиод светится только тогда, когда достигнуто максимальное значение, то есть ручка максимально повернута в одну из сторон. В остальных случаях он не светится. Будет здорово, если будут стоять несколько светодиодов, которые будут загораться и гаснуть, чтобы получилась “бегущая строка”.
Задание 2*. Необходимо подключить фоторезистор и вывести показания в монитор порта. Почти наверняка будет присутствовать шум. Отфильтруйте его любым из способов, и посмотрите в каком диапазоне значений работает фоторезистор. Отмасштабируйте его от 0 до 10.

Задание 1:

Код программы:

int sensorPin = A2;   // select the input pin for the potentiometer

int sensorValue = 0;  // variable to store the value coming from the sensor

void setup() {

// declare the ledPin as an OUTPUT:

pinMode(2, OUTPUT);

pinMode(3, OUTPUT);

pinMode(4, OUTPUT);

pinMode(5, OUTPUT);

pinMode(6, OUTPUT);

pinMode(7, OUTPUT);

pinMode(8, OUTPUT);

pinMode(9, OUTPUT);

}

void loop() {

// read the value from the sensor:

sensorValue = analogRead(sensorPin);

if (sensorValue==(0*128))

{digitalWrite(2, LOW);

digitalWrite(3, LOW);

digitalWrite(4, LOW);

digitalWrite(5, LOW);

digitalWrite(6, LOW);

digitalWrite(7, LOW);

digitalWrite(8, LOW);

digitalWrite(9, LOW);}

else if (sensorValue<=(1*128))

{digitalWrite(2, HIGH);

digitalWrite(3, LOW);

digitalWrite(4, LOW);

digitalWrite(5, LOW);

digitalWrite(6, LOW);

digitalWrite(7, LOW);

digitalWrite(8, LOW);

digitalWrite(9, LOW);}

else if (sensorValue<=(2*128))

{digitalWrite(2, HIGH);

digitalWrite(3, HIGH);

digitalWrite(4, LOW);

digitalWrite(5, LOW);

digitalWrite(6, LOW);

digitalWrite(7, LOW);

digitalWrite(8, LOW);

digitalWrite(9, LOW);}

else if (sensorValue<=(3*128))

{digitalWrite(2, HIGH);

digitalWrite(3, HIGH);

digitalWrite(4, HIGH);

digitalWrite(5, LOW);

digitalWrite(6, LOW);

digitalWrite(7, LOW);

digitalWrite(8, LOW);

digitalWrite(9, LOW);}

else if (sensorValue<=(4*128))

{digitalWrite(2, HIGH);

digitalWrite(3, HIGH);

digitalWrite(4, HIGH);

digitalWrite(5, HIGH);

digitalWrite(6, LOW);

digitalWrite(7, LOW);

digitalWrite(8, LOW);

digitalWrite(9, LOW);}

else if (sensorValue<=(5*128))

{digitalWrite(2, HIGH);

digitalWrite(3, HIGH);

digitalWrite(4, HIGH);

digitalWrite(5, HIGH);

digitalWrite(6, HIGH);

digitalWrite(7, LOW);

digitalWrite(8, LOW);

digitalWrite(9, LOW);}

else if (sensorValue<=(6*128))

{digitalWrite(2, HIGH);

digitalWrite(3, HIGH);

digitalWrite(4, HIGH);

digitalWrite(5, HIGH);

digitalWrite(6, HIGH);

digitalWrite(7, HIGH);

digitalWrite(8, LOW);

digitalWrite(9, LOW);}

else if (sensorValue<=(7*128))

{digitalWrite(2, HIGH);

digitalWrite(3, HIGH);

digitalWrite(4, HIGH);

digitalWrite(5, HIGH);

digitalWrite(6, HIGH);

digitalWrite(7, HIGH);

digitalWrite(8, HIGH);

digitalWrite(9, LOW);}

else if (sensorValue<=(8*128))

{digitalWrite(2, HIGH);

digitalWrite(3, HIGH);

digitalWrite(4, HIGH);

digitalWrite(5, HIGH);

digitalWrite(6, HIGH);

digitalWrite(7, HIGH);

digitalWrite(8, HIGH);

digitalWrite(9, HIGH);}

}

Фото бегущей строки:

Работа устройства показана на видео string.mp4

Задание 2:

Фоторезистор с резистором 10 кОм.

Код измерителя:

const int analogInPin = A2;  // Analog input pin that the potentiometer is attached to

int sensorValue = 0;  // value read from the pot

void setup() {

// initialize serial communications at 9600 bps:

Serial.begin(9600);

}

void loop() {

// read the analog in value:

sensorValue = analogRead(analogInPin);

// print the results to the Serial Monitor:

Serial.println(sensorValue);

// wait 2 milliseconds before the next loop for the analog-to-digital

// converter to settle after the last reading:

delay(2);

}

График с шумами:

Код с усредняющим фильтром:

const int analogInPin = A2;  // Analog input pin that the potentiometer is attached to

int sensorValue = 0;  // value read from the pot

int outputValue = 0;  // value output to the PWM (analog out)

void setup() {

// initialize serial communications at 9600 bps:

Serial.begin(9600);

}

void loop() {

// read the analog in value:

sensorValue = analogRead(analogInPin);

outputValue = sensorValue;

for (int i = 0; i < 1000; i++)

{ sensorValue = analogRead(analogInPin);

outputValue = (outputValue+sensorValue)/2;}

Serial.println(outputValue);

}

Диапазон входных значений:

Код с масштабированием выходных значений:

const int analogInPin = A2;  // Analog input pin that the potentiometer is attached to

int sensorValue = 0;  // value read from the pot

int outputValue = 0;  // value output to the PWM (analog out)

void setup() {

// initialize serial communications at 9600 bps:

Serial.begin(9600);

}

void loop() {

// read the analog in value:

sensorValue = analogRead(analogInPin);

outputValue = sensorValue;

for (int i = 0; i < 1000; i++)

{ sensorValue = analogRead(analogInPin);

outputValue = (outputValue+sensorValue)/2;}

// map it to the range of the analog out:

outputValue = map(outputValue, 700, 950, 0, 10);

// print the results to the Serial Monitor:

Serial.println(outputValue);

}

Работа кода показана на видео graph.mp4